# 1,1,2,2-Tetrabromoethane 
Health-Based Assessment and Recommendation 
for HEAC

Prepared by

James Unmack, MS, CIH

# 1,1,2,2-TETRABROMOETHANE

CAS 79-27-6

Document Prepared and Submitted as of

January 23, 2010

Revised April 13, 2010

Revised July 12, 2010

(latest revisions shown with yellow highlight)

DRAFT 2 
1,1,2,2-Tetrabromoethane 
Health-Based Assessment and Recommendation for HEAC 
Prepared by James Unmack, MS, CIH

## TABLE OF CONTENTS

TABLE OF CONTENTS	2

I.	IDENTIFICATION	3

II.	CHEMICAL AND PHYSICAL PROPERTIES	3

III.	USES / APPLICATIONS / OCCURRENCE / EXPOSURES	4

IV.	CURRENT EXPOSURE GUIDELINES	5

V.	PRODUCTION INFORMATION	6

VI.	MEASUREMENT INFORMATION	6

VII.	ORGANIZATIONAL SOURCES AND RECOMMENDATIONS	6

VIII.	ODOR THRESHOLDS	7

IX.	HEAC HEALTH-BASED ASSESSMENT AND RECOMMENDATION	7

X.	HEALTH HAZARD DATA	8

XI.	NOTABLE STUDIES	9

XII.	REFERENCES	10

1,1,2,2-Tetrabromoethane 
Health-BasedAssessment and Recommendation for HEAC 
Prepared by James Unmack, MS, CIH

For discussion March 24, 2010

## IDENTIFICATION

Substance Name	1,1,2,2-TETRABROMOETHANE

CAS	79-27-6

Synonyms	Acetylene tetrabromide, ethane tetrabromide, muthmann’s liquid, tetrabromoethane, sym-tetrabromoethane, tetrabromoacetylene, TBE

Molecular Formula	C2H2Br4

Structural Formula	CHBr2CHBr2

## CHEMICAL AND PHYSICAL PROPERTIES

Physical state and appearance	Colorless to yellowish liquid

Odor description	Sweetish,but pungent odor,

Odor threshold	1 - 2 ppm, apparent and objectionable

Molecular weight	345.65328 daltons

ppm to mg/m³	conversion factor at 25C and 760 mmHg

1 ppm = 14.14 mg/m³ and 1 mg/m³ = 0.0707 ppm

Vapor Pressure at 24C	5.32 Pa @ 24C (52.5 ppm saturated vapor)

(0.02 - 0.1 mmHg @ 20 C) (26 - 130 ppm)

Viscosity	9 centipoise

Relative Vapor Density	11.9 (air = 1)

Melting point	0 C, 273 K

(-1 – 1 C, 30 – 34 F)

Boiling point	243.5 C, 516.7 K

(135 – 244 C, 275 – 471 F)

Specific Gravity	2.9656 at 20 C

Solubility	0.678 g/L water at 25 C

Miscible in ethanol, chloroform, ether, aniline, glacial acetic acid

Corrosivity 	Will attack some plastics, rubbers, and coatings

Special Characteristics	Very dense liquid.

Flammability	Not flammable.

Other Hazards	When heated to 190 C it decomposes to toxic fumes of carbonyl bromide, hydrogen bromide, and carbon monoxide.

Chemically incompatible with active metals such as aluminum, magnesium, and zinc, hot iron, and strong caustics

## USES / APPLICATIONS / OCCURRENCE / EXPOSURES

Uses & Applications: Tetrabromoethane is used as a solvent, a gauge fluid and substitute for mercury, and an ore flotation agent for separating dense ore from lighter supporting rock. Sand, limestone, dolomite, and other types of rock material will float while minerals such as sphalerite, galena, and pyrite will sink. It is use as an index of refraction fluid in optical microscopy, a catalyst in synthetic fiber production, and an additive for flame-proofing polyurethanes, polyolefins, and polystyrenes.

Occurrence & Exposures: Damage or leakage of equipment containing tetrabromoethane.

## CURRENT EXPOSURE GUIDELINES

8 CCR 5155, Table AC-1	1 ppm, 14 mg/m³

29 CFR 1910.1000	1 ppm, 14 mg/m³

ACGIH TLV	0.1 ppm, 1.4 mg/m3

NIOSH REL	Not established

NIOSH IDLH	8 ppm (112 mg/m³) (8/16/1986)

EPA RfC	0 Not established

Inhalation Reference Concentration

OEHHA aREL	Not established

OEHHA cREL	Not established

Other OELs:	1 ppm TWA

Australia

1 ppm TWA

Belgium

1 ppm TWA

Denmark

1 ppm TWA

Finland

1 ppm TWA

France

1 ppm MAK

Germany

7 mg/m3 MAC TGG

Netherlands

0.5 ppm TWA skin

United Kingdom

1 ppm TWA

Philippines

1 ppm TWA, 2 ppm STEL

Sweden

1 ppm TWA

Turkey

## PRODUCTION INFORMATION

Three manufacturers identified, two in China, one in Israel. Available in technical and reagent grade.

## MEASUREMENT INFORMATION

NIOSH 2003 solid sorbent (silica gel), gas chromatograph with FID

The NIOSH Manual of Analytical Methods (NMAM) lists the detection limit for this method as 0.08 mg per sample. Taking the maximum recommended sample size of 100 liters (0.1 m³) collected at 0.2 to 1 liter per minute translates to 0.8 mg/m³ or approximately 0.06 ppm in as little as 100 minutes.

An analytical method using an electron capture detector, such as NIOSH 5602, would have a detection limit of 0.05 to 0.2 µg per sample allowing measurements of airborne concentrations in the low parts per billion range.

## ORGANIZATIONAL SOURCES AND RECOMMENDATIONS

ACGIH	TLV	0.1 ppm TWA (2006)

TLV	2 ppm ceiling (2003)

TLV	5 ppm ceiling (1963 – 2003)

TLV	5 ppm TWA (1948 – 1962)

MAC	10 PPM TWA (1946-1947)

AIHA	Emergency Response Planning Guideline (ERPG):

No ERPG published for tetrabromoethane

AIHA	Workplace Environmental Exposure Level (WEEL):

No WEEL published for tetrabromoethane

EPA	Mutagenic by Bruce Ames test

NTP	Not studied for cancer

IARC	Not studied

Proposition 65	Not listed

## ODOR THRESHOLDS

None published. Odor reported to be sweetish, but pungent, readily apparent and objectionable.

## HEAC HEALTH-BASED ASSESSMENT AND RECOMMENDATION

HEAC recommendation for PEL 0.03 ppm TWA (0.42 mg/m3), Inhalable fraction and vapor

This recommendation is based on avoiding acute irritation of the upper respiratory tract, pulmonary edema and hepatic effects. Although 1,1,2,2-tetrabromoethane tested positive for mutagenic effects with the Bruce Ames test using Salmonella tiphimurium, insufficient information is available to rate its carcinogenic potential.

Acute toxicity studies found an oral LD50 for rabbits and guinea pigs of 400 mg/kg and 1600 mg/kg for rats.

Chronic exposure studies, 100 to 106 days duration, with rabbits, guinea pigs, rats, mice, and a monkey found pulmonary edema and fatty degeneration of the liver at 14 ppm average concentration for 7 hours per day, 5 days per week. Guinea pigs exposed at 4 ppm for 180 days showed slight changes in lung and liver tissue. No effects were observed at 1.1 ppm in five animal species. (Hollingsworth 1963)

Transcutaneous absorption has been observed in humans and laboratory animals. Application of 15 mg to the skin produced a statistically significant increase incidence of papillomas in the forestomach of Swiss mice. (Van Duuren 1979) It was not reported whether this was due to self grooming or transcutaneous absorption.

An interspecies uncertainty factor (UF) of 3 was used because similar results were seen across five test species. Toxicologists on the HEAC recommend using a default intraspecies uncertainty factor of 10. Many industrial hygienists use a default intraspecies uncertainty factor of 3. Applying the interspecies UF of 3 and intraspecies UF of 10 to the no observed adverse effect level (NOAEL) of 1.1 ppm yields 0.03 ppm. This results in a recommended PEL one-third the ACGIH TLV.

## HEALTH HAZARD DATA

Routes of Entry	Inhalation: 	Yes

Skin:	Yes

Ingestion:	Yes1

| Table of Exposure Responses with Result and Reference |  |  |
| --- | --- | --- |
| Exposure / Exposed / Time | Result | Reference |
| 500 mg/m3, 3 hour, mouse | Behavioral, general anesthetic | 1990, Bandman |
| 549 mg/m3, 4 hour, rat | LC50 | 1982, Izmerov |
| 550 mg/m3, rat | LC50 | 1990, Bandman |
| 269 mg/kg, oral mouse | LD50 | 1982, Izmerov |
| 1200 mg/kg, oral rat | LD50 | 1990, Bandman |
| 400 mg/kg, rabbits guinea pigs | LD50 | 1950, Gray |
| 1600 mg/kg, rats | LD50 | 1950, Gray |
| 5250 mg/kg, dermal rat | LD50 | 1963, Hollingsworth |
| 14 ppm, 7 hr/day. 14 wk | Pulmonary edema, fatty liver | 1963, Hollingsworth |
| 4 ppm, 7 hr/day. 14 wk | Lung and liver changes | 1963, Hollingsworth |
| Saturated vapor (26 - 130 ppm), 7 hr, rats | Slight eye and nose irritation | 1963, Hollingsworth |
| 2 ppm with peak of 16 ppm, 10 minutes, human | Central nervous system dysfunction, solvent encephalopathy | 1969, Van Haaften |

## NOTABLE STUDIES

Table listing of studies more extensively used in these efforts.

| 1Author and Date | Study Type | Results | Discussion and Assessment |
| --- | --- | --- | --- |
| Hollingsworth RL, Rowe VK, Oyen F: Toxicity of acetylene tetrabromide determined on experimental animals, AIHAJ, 24:28-35 (1963) | Oral, eye, skin and vapor exposure studies | Low single dose toxicity. Chronic exposure to skin produce irritation, chronic inhalation produces lung and liver injury. | 5 animal species test with similar results in all species. No adverse effects observed at 1.1 ppm. |
| Gray MG, Effect of exposure to the vapors of tetrabromoethane (acetylene tetrabromide), Arch. Ind. Hyg. Occup. Med. 2:407-419 (1950) | Single oral dose, multiple species | LD50 o = 400 mg/kg for rabbits and guinea pigs LD50 = 1600 mg/kg for rats | Death was delayed 3 to 5 days following dose |
| Van Duuren BL, Goldschmidt BM, Loewengart G, et al., Carcinogenicity of halogenated olefinic and aliphatic hydrocarcarbons in mice, J Natl Cancer Inst 64(6):1433-1439 (1979) | Transcutaneous application | Increased incidence of forestomach papillomas | Biological significance of forestomach papillomas in Swiss mice is unclear. |
| Morrow LA, Callender T, Lottenberg S, et al., PET and neurobehavioral evidence of tetrabromoethane encephalopathy, J. Neuropsychiatry Clin Neurosci 2:431-435 (1990) | Study of an accidental exposure victim | Residual neurological changes resulting from single acute exposure | Transcutaneous exposure from TBE splashed on face |
| Van Haaften AB, Acute tetrabromoethane (acetylene tetrabromide) intoxication in man, AIHAL 30:251-256 (1969) | Study of accidental exposure | Long term health effects from acute exposure. | Estimated 1-2 ppm exposure for 7 hours with 10 minutes at 16 ppm. |

## REFERENCES

- Gray MG, Effect of exposure to the vapors of tetrabromoethane (acetylene tetrabromide), Arch Ind Hyg Occup Med 2:407-419 (1950)

- Hollingsworth RL, Rowe VK, Oyen F, Toxicity of acetylene tetrabromide determined on experimental animals, AIHAJ 24:28-35 (1963)

- Izmerov NF, Sanotsky IV, Sidorov KK, Toxicometric parameters of industrial toxic chemicals under single exposure, Centre of International Projects, GKNT p. 107 (1982)

- JT Baker, 1,1,2,2-Tetrabromoethane, MSDS Number T0494, July 27, 2006

- Kennedy CH, Cohen KB, Bechtold WE, et al., Effect of dose on metabolism of 1,1,2,2-tetrabromoethane in F344/N rats after gavage administration, Toxicol Appl Pharmacol 119:23-33 (1993)

- Morrow LA, Callender T, Lottenberg S, et al., PET and neurobehavioral evidence of tetrabromoethane encephalopathy, J Neuropsychiatry Clin Neurosci 2:431-435 (1990)

- NIOSH Registry of Toxic Effects of Chemical Substances, Ethane, 1,1,2,2-tetrabromo-, RTECS No. K1822500, November 2004

- Rosenkrantz HS, Mutagenicity of halogenated alkanes and their derivatives, Environ Health Perspect 21:79-84 (1977)

- Sciencelab.com, Inc., 1,1,2,2-Tetrabromoethane, MSDS SLT1357, November 6, 2008

- Strobel K, Grumm T, Aliphatic and aromatic halocarbons as potential mutagens in drinking water. III. halogenated ethanes and ethenes, Toxicol Environ Chem 15:101-128 (1987)

- U.S. National Toxicology Program, Renal toxicity studies of selected halogenated ehtanes administered by gavage to F344 rats, Toxicity Report Series 45, DHHS (NIH) Pub. No. 96-3935, NTP, Research Triangle Park, NC (1996)

- Van Haafen AB, Acute tetrabromoethane (acetylene tetrabromide) intoxication in man, AIHAJ 30:251-256 (1969)

- Van Duuren BL, Goldschmidt BM, Lowewngart G, et al., Carcinogenicity of halogenated olefinic and aliphatic hydrocarbons in mice, J Natl Cancer Inst 64(6):1433-1429 (1979)