# N-PROPANOL 
Health-Base Assessment and Recommendation 
for HEAC

Prepared by

James L. Unmack, MS, CIH

for discussion December 14, 2010

# n-PROPANOL

CAS Number 71-23-8

Document prepared and submitted as of

October 24, 2010

## DRAFT 1 
n-PROPANOL

### Health-Based Assessment and Recommendation for HEAC 
Prepared by James L. Unmack, MS, CIH

for discussion December 15, 2010

### Table of Contents

I.	IDENTIFICATION	1

II.	CHEMICAL AND PHYSICAL PROPERTIES	1

III.	USES, APPLICATIONS, OCCURRENCE, EXPOSURES	2

IV.	CURRENT EXPOSURE GUIDELINES	2

V.	PRODUCTION INFORMATION	2

VI.	MEASUREMENT INFORMATION	2

VII.	ORGANIZATIONAL SOURCES AND RECOMMENDATIONS	2

VIII.	ODOR THRESHOLD	3

IX.	HEAC HEALTH BASED ASSESSMENT AND RECOMMENDATIOIN
	3

X.	HEALTH HAZARD DATA	4

XI.	NOTABLE STUDIES	4

XII.	REFERENCES	4

## DRAFT 1 
n-PROPANOL

### Health-Based Assessment and Recommendation for HEAC 
Prepared by James L. Unmack, MS, CIH

#### IDENTIFICATION

Substance Name:	n-propanol

CAS Number:	71-23-8

Synonyms:	Ethyl carbinol; Propan-1-ol; 1-Propanol; n-Propyl Alcohol

Molecular Formula:	C3H8O

Structural Formula:	CH3CH2CH2OH

#### CHEMICAL AND PHYSICAL PROPERTIES

Physical State and Appearance	Clear, colorles liquid with a sweet odor

Odor Threshold	2.6 ppm to 40 ppm

Molecular Weight	60.09 daltons

Specific Gravity	0.8053 at 20̊C

Melting Point	-127̊C

Boiling Point	97.2̊C

Vapor Pressure	15 torr at 20̊C

Flash Point	15̊C, closed cup

Explosive Limits	UEL 13.5% , LEL 2.1% by volume in air

Solubility	Miscible in water

Very soluble in alcohol, ether

Conversion Factors at 760 torr, 25̊C

1 ppm = 2.46 mg/m³; 1 mg/m³ = 0.4 ppm

#### USES, APPLICATIONS, OCCURRENCE, EXPOSURES

##### Uses and Applications

n-Propanol is used as a solvent for waxes, vegetable oils, resins, cellulose esters, and ethers. It is found in brake fluids and polishing compounds and has been used as a degreasing agent, an antiseptic, and a chemical intermediate. More recently, it is being used as a hand disinfectant by health care workers.

##### Occurrence and Exposures

It has been measured in airborne exposures during label printing and production operations.

#### CURRENT EXPOSURE GUIDELINES

8 CCR 5155	200 ppm TWA, 250 ppm STEL

500 mg/m³ TWA, 625 mg/m³ STEL

29 CFR 1910.1000		200 ppm TWA, 250 ppm STEL

ACGIH TLV	100 ppm TWA, A-4 not classifiable as a Human Carcinogen

NIOSH REL	200 ppm TWA, 250 ppm STEL

DFG MAK	200 ppm TWA, 250 ppm STEL

#### PRODUCTION INFORMATION

TBD

#### MEASUREMENT INFORMATION

NIOSH 1405 method: collect on coconut shell charcoal, analyze by GC/FID. Listed limit of detection (LOD): 1 ug sample so 15-minute STEL sampling at maximum flow rate of 0.2 liters per minute yields 15-min LOD of 0.001mg/0.003m3 (0.33 mg/M3 or 0.13 ppm), well below STEL of 250 ppm.

#### ORGANIZATIONAL SOURCES AND RECOMMENDATIONS

ACGIH	1966 proposed TLV 200 ppm TWA

1968 - 2006 TLV 200 ppm TWA

1974 - 2003 Skin notation

1976 – 2003 TLV 250 ppm STEL

1999 proposed A-3 confirmed animal carcinogen, with unkown relevance to humans, withdraw skin notation

2002 proposed TLV 200 ppm TWA, A3, no STEL

2003 proposed TLV 200 ppm TWA, 400 ppm STEL, A3, withdraw Skin notation

2004 TLV 200 ppm TWA, 400 ppm STEL, A3, withdraw Skin notation

2005 propose TLV 100 ppm TWA, 200 ppm STEL, A4

2006 propose TLV 100 ppm TWA, no STEL, A4

2007 TLV 100 ppm TWA, A4

AIHA	 no WEEL published for n-propanol

AIHA	 no ERPG published for n-propanol

#### ODOR THRESHOLD

Critiqued sources:	0.08 ppm - 269 ppm

Secondary sources:	1.1 ppm - 10,172 ppm

#### HEAC HEALTH BASED ASSESSMENT AND RECOMMENDATIOIN

Recommendation for 100 ppm TWA (250 mg/m³ TWA)

This recommendation is based on avoiding acute sensory irritation. In acute animal studies, n-propanol is more toxic than 2-propanol (IPA). The respiratory depression test (RD50) gave results that varied with the strain of the test animal and the duration of the exposure. Values of RD50 varied from 4,780 ppm for 5 minutes to 13,660 ppm for 30 minutes. Suggestions for an occupational exposure limit based on the RD50, run from 1% to 10% of the RD50, giving a range of 50 ppm to 400 ppm. The geometric mean of this range is 141 ppm, which is then rounded to 100 ppm. No data exist upon which to base a sensitizer (SEN) or Skin notation or to recommend a STEL.

#### HEALTH HAZARD DATA

Many acute toxicity studies have been conducted with n-propanol with findings of acute LD50 ranging from 1.9 g/kg to 6.5 g/kg. Inhalation studies find acute LC50 ranging from 4,100 ppm to 24,500 ppm.

n-Propanol is not irritating to the skin of rabbits. Dermal toxicity ranged from 4 g/kg to 6.7 g/kg. The US EPA measured human skin permeability to range from 1.3 x 10-3 cm/hour to 1.7 x 10-3 cm/hour.

n-Propanol yielded negative results in genotoxicity studies. Reproductive and developmental toxicity studies found exposures of 7000 to 10,000 ppm 7 hours per day found maternal toxicity and reduced mean fetal body weights. Exposure to 3,500 ppm produced no evidence of maternal toxicity or developmental impairment. The no-observed-adverse-effect level (NOAEL) for developmental toxicity in rats was 3,500 ppm.

Human exposure studies found eye and nose irritation in the range of 4,000 to 16,000 ppm. One fatality was attributed to acute ingestion of n-propanol.

#### NOTABLE STUDIES

To be added.

#### REFERENCES

To be added.