Substance name:	trichloroethylene

Reviewer:  Will Forest, 3/11/09

CAS:  79-01-06		MW:  131.4		Synonyms:	TCE; trichloroethene; trichlor; tri

Molecular formula:  C2HCl3		Structural formula:  CHCl=CCl2

Conversion:  1 ppm = 5.37 mg/M3	and	1 mg/M3 = 0.19 ppm             at 25 oC and 760 mm/Hg

Vapor pressure = 77 torr (about 100,000 ppm) at 25 oC

Physical characteristics at room temp:  volatile colorless liquid

Special physical characteristics if any:  vapor density = 4.5 (air = 1)

Flammability and other hazards:  combustion products include phosgene, HCl, and perhaps dioxin-like compounds

Uses/applications:  solvent – primarily in degreasing, also in the rubber and paper industries and in adhesives; ingredient in some pesticides

## Standards and Recommendations

Title 8 PEL:		25 ppm; 100 ppm STEL; 300 ppm ceiling

ACGIH TLV (2007):   10 ppm; 25 ppm STEL

NIOSH REL (1996):    25 ppm (except 2 ppm 60-minute “ceiling” during anesthetic use)

Other OELs:  WEEL (none?), MAK (none?)

## Organizational assessments

IARC (1995):  Probable Human Carcinogen 
(sufficient animal evidence, limited human evidence)

NTP (11th Report, 2005):  Reasonably Anticipated to be a Human Carcinogen
(sufficient animal evidence, limited human evidence)

EPA (2001):  Highly Likely to Produce Cancer in Humans,   or   B1 Probable Human Carcinogen
(sufficient animal evidence, limited human evidence), depending on guidelines used

European Union:  Category 2

(substances which should be regarded as if they are carcinogenic to man)

Prop 65 (1988):  Known to Cause Cancer

OEHHA NSRLs:       80 g/day (inhalation); 50 g/day (oral)

OEHHA Inhalation Unit Risk:        .002 (mg/m3)-1

OEHHA Inhalation Slope Factor:   .007 (mg/kg/day)-1 (revised down from .01 on 12/20/02)

OEHHA Oral Slope Factor:  0.013 (mg/kg-day)-1 (revised from .0153 (mg/kg-day)-1on 9/24/03)

OEHHA Chronic REL:  0.1 ppm = 0.6 mg/m3, based on eye irritation, drowsiness, heart palpitations, cough, weakness, and dizziness in workers (Vandervort & Polnkoff, 1973)

OEHHA Public Health Goal (1999):  .0008 mg/L (i.e., .8 ppb)

DHS MCL (updated 2004):  .005 mg/L (i.e., 5 ppb – the federal MCL)

## Peer-reviewed journal articles and other studies

### Krishnadasan 2007

nested case-control prostate cancer study in American aerospace workers; 362 cases, 1805 matched controls, 1950-1992; job-exposure matrix exposure assessment

low-TCE-exposure group had OR 1.3; high-exposure group OR 2.1, significant; positive trend, p=.02

### Bruning 2003

German hospital-based case-control study; 134 renal cell cancer (RCC) cases and 401 controls

exposure was self-assessed and job history was analyzed, using expert-based exposure information

OR 1.80 (CI 1.01-3.20) for longest-held job in TCE-exposing industries

OR 5.57 (CI 2.33-13.32) any exposure in “metal degreasing”

OR 3.71 (CI 1.80-7.54) self-reported narcotic symptoms, indicative of peak exposures

### Sun 2007

cohort study on breast cancer; 23-year follow-up; 286 cases

63,982 female former workers at a Taiwanese electronics factory, 1973-1997, compared to gen’l population

SIR close to 1

SIR 1.38 (1.11-1.70) for cohort first employed before 1974

SIR 1.62 for 10 years of employment

### Charbotel 2006

French case-control renal cell cancer study; 86 cases and 316 controls matched for age and gender

three types of exposure assessment:  exposure for at least one job period (minimum 1 year); cumulative dose (ppm TCE * years in job period); and exposure to peaks.

OR = 2.16 (1.02-4.60) for high cumulative doses

dose-response relationship

OR for highest exposure 2.73 (1.06-7.07)

after adjusting for exposure to cutting fluids, ORs remained high but were not significant

### Raaschou-Nielsen 2003

cohort study of 40,049 blue-collar workers in 347 Danish companies, 1968-1997

SIRs for total cancer 1.1 (1.04, 1.12) in men and 1.2 (1.14, 1.33) in women

SIRs for non-Hodgkin's lymphoma and renal cell carcinoma 1.2 (1.0, 1.5) and 1.2 (0.9, 1.5), respectively

SIRs increased with duration of employment, and elevated standardized incidence ratios were limited to workers first employed before 1980 for non-Hodgkin's lymphoma and before 1970 for renal cell carcinoma

SIR for esophageal adenocarcinoma 1.8 (1.2, 2.7); higher in companies with the highest probability of TCE exposure

In a subcohort of 14,360 presumably highly exposed workers, the SIRs for non-Hodgkin's lymphoma, renal cell carcinoma, and esophageal adenocarcinoma were 1.5 (95% CI: 1.2, 2.0), 1.4 (95% CI: 1.0, 1.8), and 1.7 (95% CI: 0.9, 2.9), respectively.

### Hansen 2001

cancer incidence study; 803 Danish workers; historical files of individual air and urinary measurements of TCE exposure

SIR for cancer overall not elevated

SIR = 3.5 for non-Hodgkin's lymphoma in men, significant

SIR = 4.2 for cancer of the esophagus in men, significant

SIR = 3.8 for cervical cancer in women, significant

no clear dose-response relationship

no increased risk for kidney cancer

### Zhao 2005

retrospective cancer incidence and mortality cohort studies of 6,107 male aerospace workers

employed for at least two years between 1950 and 1980, followed up until 2001

exposure to TCE and other substances assessed by job exposure matrix (none, low, medium, or high)

RR = 4.90 (1.23 - 19.6) for kidney cancer incidence in high exposure group

RR = 1.87 (0.56 - 6.20) for kidney cancer incidence in medium exposure group

P = 0.023 for kidney cancer incidence trend

RR = 2.03 (0.50 - 8.32) for kidney cancer mortality in high exposure group

RR = 1.43 (0.49 - 4.16) for kidney cancer mortality in medium exposure group

P = 0.307 for kidney cancer mortality trend

RR = 1.98 (0.93 - 4.22) for bladder cancer incidence in high exposure group

### Dubosq 2005

case study; 51-year-old man with renal cell carcinoma and a history of TCE abuse

## HEAC Health-based assessment and  recommendation

Most sources agree that TCE should be considered a probable human carcinogen, based on considerable human as well as extensive animal evidence.  OEHHA’s inhalation No Significant Risk Level, calculated to induce one excess cancer per 100,000 persons exposed full-time for a lifetime, is 80 g/day.  Here’s one way to describe the calculation of a workplace exposure limit that should pose a 1/1000 risk:

To go from risk of 1/100,000 to risk of 1/1000:

80 g/day * 100  =  8000 g/day, or  8 mg/day

To condense that lifetime daily exposure into a working lifetime daily exposure:

8 mg/day * 70/40 (years/lifetime) * (52/50 weeks/year) * 7/5 (days/week)   =   20.384 mg/day

Assuming inhalation of 10 m3/day:

20.384 mg/day / 10 m3/day  =  2.0384 mg/m3

By the conversion factor:

2.0384 mg/m3 * 1 ppm/5.37 mg/m3  =  0.38 ppm

Thus, linear calculation from the OEHHA NSRL indicates that the working lifetime exposure associated with a 1/1000 cancer risk is 0.38 ppm.

From OEHHA’s inhalation Unit Risk Factor of .002 (mg/m3)-1, I did this comparable calculation:

Working lifetime exposure / lifetime exposure  =  40/70 * 50/52 * 5/7 * 10/20  =  0.196

Working lifetime fraction * Unit Risk Factor  =  .196 * .002 (mg/m3)-1  =  .000392 (mg/m3)-1

By the conversion factor,   .000392 (mg/m3)-1  =  .002105 ppm-1

So, the PEL for a cancer risk level of 1/1000 is   .001 / .002105 ppm-1  =  .475 ppm

OEHHA’s Chronic REL is based on a worker LOAEL, with an average exposure (for “approximately 100 ‘affected employees’”) of 32 ppm, 8 hours/day, 5 days/week, over an 8-year period.  OEHHA used uncertainty factors of 10 for the use of a LOAEL, and 10 for intraspecies variation, for a cumulative uncertainty factor of 100.  OEHHA transformed the exposure periods to environmental periods, but since we’re looking at workplace exposure we don’t need to; we simply divide the 32-ppm exposure by 100, giving 0.32 ppm.

HEAC Reviewer responses to comments of the Halogenated Solvents Industy Alliance (HSIA) on TCE cancer risk assessment in their letter of May 23, 2008  (icon link may take a few seconds to appear below):

## Usage information:

High Volume Chemical, over a million pounds per year produced (~6 million pounds per year released or disposed of on-site or off-site, per TRI)

## Measurement information

OSHA Method  -  Lists reliable limit of quantitation at 0.81 ug/sample or 0.013 ppm (13 ppb) in a 240 minute sample   http://www.osha.gov/dts/sltc/methods/mdt/mdt1001/1001.html

NIOSH Manual of Analytical Methods 1022 – estimated LOD 0.01 mg/sample translates to 0.067 ppm

(67 ppb) in the maximum recommended 30 liter sample at 0.2 liter/minute flow rate

http://www.cdc.gov/niosh/nmam/pdfs/1022.pdf

## References cited or reviewed

U.S. National Academy of Sciences (NAS). 2006. Assessing Human Health Risks of Trichloroethylene – Key Scientific Issues. Committee on Human Health Risks of Trichloroethylene, National Research Council.

http://books.nap.edu/catalog.php?record_id=11707

OEHHA.  May 2005.  Technical Support Document for Describing Available Potency Factors.  pp. B-525 - B-533.

http://www.oehha.org/air/hot_spots/pdf/May2005Hotspots.pdf

OEHHA.  May 2008.  Proposition 65 Safe Harbor Levels:  No Significant Risk Levels for Carcinogens and Maximum Allowable Dose Levels for Chemicals Causing Reproductive Toxicity.   http://www.oehha.org/prop65/pdf/2008MayStatusReport.pdf

U.S. Environmental Protection Agency (USEPA).  2001.  Trichloroethylene Health Risk Assessment: Synthesis and Characterization (External Review Draft)

http://cfpub.epa.gov/ncea/cfm/recordisplay.cfm?deid=23249

Trichloroethylene Health Risks – State of the Science.

Environmental Health Perspectives Supplements Volume 108, Number S2, May 2000.

http://www.ehponline.org/docs/2000/suppl-2/toc.html

IARC. Dry Cleaning, Some Chlorinated Solvents and Other Industrial Chemicals. In: IARC Monographs on the Evaluation of Carcinogenic Risks to Humans, Vol 63. Lyon:International Agency for Research on Cancer, 1995.

U.S. National Toxicology Program (NTP). 2005. Trichloroethylene, in the 11th Annual Report on Carcinogens.

http://ntp.niehs.nih.gov/ntp/roc/eleventh/profiles/s180tce.pdf

Agency for Toxic Substances and Disease Registry (ATSDR). 1997. Toxicological Profile for Trichloroethylene.

http://www.atsdr.cdc.gov/toxprofiles/tp19.pdf

OEHHA.  Trichloroethylene Chronic Toxicity Summary.

http://www.oehha.ca.gov/air/chronic_rels/pdf/79016.pdf

OEHHA. 1999. Public Health Goal for Trichloroethylene In Drinking Water.

http://www.oehha.org/water/phg/pdf/tce_f.pdf

California Air Resources Board. 1990. Proposed Identification of Trichloroethylene as a Toxic Air Contaminant.

http://www.oehha.ca.gov/air/toxic_contaminants/pdf1/trichloroethylene.pdf

NIOSH Safety and Health Topic:  Trichloroethylene.  Web page.

http://www.cdc.gov/niosh/topics/trichloroethylene/

NIOSH. 1996. Trichloroethylene IDLH Documentation.

http://198.246.98.21/niosh/idlh/79016.html

Lappé and Chalfin.  Identifying Toxic Risks Before and During Pregnancy:  A Decision Tree and Action Plan.  A Report to the March of Dimes.  April 15, 2002.

http://environmentalcommons.org/cetos/articles/MoDFinalReport.pdf

Krishnadasan et al.  Nested case-control study of occupational chemical exposures and prostate cancer in aerospace and radiation workers.  Am J Ind Med. 2007, May; 50(5):383-90.

Sung TI; Chen PC; Jyuhn-Hsiarn Lee L; Lin YP; Hsieh GY; Wang JD.  Increased standardized incidence ratio of breast cancer in female electronics workers.  BMC Public Health. 2007; 7(147):102.

Bruning T; Pesch B; Wiesenhutter B; Rabstein S; Lammert M; Baumuller A; Bolt HM.  Renal cell cancer risk and occupational exposure to trichloroethylene: results of a consecutive case-control study in Arnsberg, Germany.  Am J Ind Med. 2003, Mar; 43(3):274-85.

Raaschou-Nielsen O Hansen J McLaughlin JK Kolstad H Christensen JM Tarone RE Olsen JH.  Cancer risk among workers at Danish companies using trichloroethylene: a cohort study.  Am J Epidemiol. 2003, Dec 15; 158(12):1182-92.

Hansen J; Raaschou-Nielsen O; Christensen JM; Johansen I; McLaughlin JK; Lipworth L; Blot WJ; Olsen JH.  Cancer incidence among Danish workers exposed to trichloroethylene.  J Occup Environ Med. 2001, Feb; 43(2):133-9.

Charbotel B; Fevotte J; Hours M; Martin JL; Bergeret A.  Case-control study on renal cell cancer and occupational exposure to trichloroethylene. Part II: Epidemiological aspects.  Ann Occup Hyg. 2006, Nov; 50(8):777-87.

Zhao Y; Krishnadasan A; Kennedy N; Morgenstern H; Ritz B.  Estimated Effects of Solvents and Mineral Oils on Cancer Incidence and Mortality in a Cohort of Aerospace Workers.  American Journal of Industrial Medicine 2005, 48:249-258.

Dubosq F; Linke C; Cardot V; Meria P; Desgrandchamps F; Teillac P.  [Renal cell carcinoma and trichloroethylene].  Prog Urol. 2005, Jun; 15(3):525-8.